M2: Formal LIMO ISA Specification (Frozen)

1. Architectural State (Registers)

LIMO features a 32-register file (R0–R31). This preserves the 5-bit register fields standard in RV32I, ensuring easy translation to Ripes for testing.

| Number | Sesotho Name | Alias | Type | Purpose |
| --- | --- | --- | --- | --- |
| R0 | Lefela | Zero | SPR | Hard-wired to 0. Writes ignored. |
| R1 | Aterese ea Khutlo | Ra | SPR | Return address. |
| R2 | Sesupa Pokello | Sp | SPR | Stack pointer. |
| R3 | Sesupa Kakaretso | Gp | SPR | Global pointer. |
| R4 | Sesupa Mola | Tp | SPR | Thread pointer. |
| R5-R7 | Nakoana 1-3 | t0-t2 | GPR | Temporaries. |
| R8-R9 | Bolokiloeng 1-2 | s0-s1 | GPR | Saved registers. |
| R10-R17 | Khang 1-8 | a0-a7 | GPR | Function arguments / returns. |
| R18-R27 | Bolokiloeng 3-12 | s2-s11 | GPR | Saved registers. |
| R28-R31 | Nakoana 4-7 | t3-t6 | GPR | Temporaries. |
| PC | Sesupa Taelo | pc | SPR | Program Counter. |

2. Instruction Formats

LIMO uses fixed 32-bit instructions. The field widths match RV32I exactly (5-bit rd, rs1, rs2).

- R-Type: funct7 (7) | rs2 (5) | rs1 (5) | funct3 (3) | rd (5) | opcode (7)

- I-Type: imm[11:0] (12) | rs1 (5) | funct3 (3) | rd (5) | opcode (7)

- S-Type: imm[11:5] (7) | rs2 (5) | rs1 (5) | funct3 (3) | imm[4:0] (5) | opcode (7)

- B-Type: imm[12|10:5] (7) | rs2 (5) | rs1 (5) | funct3 (3) | imm[4:1|11] (5) | opcode (7)

- J-Type: imm[20|10:1|11|19:12] (20) | rd (5) | opcode (7)

3. Opcode and Function-Field Assignments (The Assembler Blueprint)

This is the exact mapping Setente needs for the assembler logic.

| Format | Opcode | Mnemonic | funct3 | funct7 | RV32I Equivalent |
| --- | --- | --- | --- | --- | --- |
| R | 0110011 | kpn (add) | 000 | 0000000 | add |
| R | 0110011 | tls (sub) | 000 | 0100000 | sub |
| R | 0110011 | le (and) | 111 | 0000000 | and |
| R | 0110011 | kp (or) | 110 | 0000000 | or |
| R | 0110011 | kpe (xor) | 100 | 0000000 | xor |
| R | 0110011 | stts (sll) | 001 | 0000000 | sll |
| R | 0110011 | stto (srl) | 101 | 0000000 | srl |
| I | 0010011 | kpnh (addi) | 000 | — | addi |
| I | 0010011 | leh (andi) | 111 | — | andi |
| I | 0010011 | kph (ori) | 110 | — | ori |
| I | 0010011 | kpeh (xori) | 100 | — | xori |
| I | 0000011 | tsl (lw) | 010 | — | lw |
| S | 0100011 | bl (sw) | 010 | — | sw |
| B | 1100011 | khl (beq) | 000 | — | beq |
| B | 1100011 | khsl (bne) | 001 | — | bne |
| B | 1100011 | khn (blt) | 100 | — | blt |
| B | 1100011 | khk (bge) | 101 | — | bge |
| I | 1100111 | tlkr (jalr) | 000 | — | jalr |
| J | 1101111 | tlk (jal) | — | — | jal |
| J | 1101111 | tl (j) | — | — | jal x0, offset |

M2 Sprint Note / D1 Feedback Log

Role: ISA Architect
Status: ISA Frozen for M2.
D1 Feedback Applied:
During D1, the examiner noted that our ISA completely drops RV32I's immediate shift instructions (slli, srli).
Decision: We have retained the original M1 decision to only use register-based shifts (stts, stto). This simplifies the datapath's shifter logic (the ALU only needs to read rs2 for shifts, and we don't need extra multiplexing for shamt). We accept that software must load the shift amount into a register first.

Handover to Team:

- To Setente (Assembler): The opcodes above are absolute. You can hardcode these into your string-matching logic. Note the exact casing (lowercase mnemonics).

- To Leputla (Pipeline): The ISA uses 5-bit register fields (rs1, rs2, rd). The Branch and Jump instructions resolve in the EX stage.

- To Thulo (Test): Use the exact hand-encoded binaries from the M1 PDF (Section 2.5) as your baseline unit tests for the assembler.