A1 — Machine Style

32-bit processor — The processor works with 32-bit values and uses 32-bit registers and data paths.

Load/store architecture — Only load and store instructions access memory. Arithmetic and logical instructions operate on registers.

Byte-addressed — Each memory address refers to one byte of memory.

Little-endian — When a 32-bit word is stored in memory, the least significant byte is stored at the lowest address.

Word-aligned — A 32-bit word must be stored at an address that is a multiple of 4.

32-bit fixed-length instructions — Every instruction is exactly 32 bits long, making instruction fetching and decoding simpler.

R, I, S and B formats — LIMO uses these instruction formats for different operations:
• R-type: register-to-register operations.
• I-type: immediate operations, loads and register-based jumps.
• S-type: store instructions.
• B-type: conditional branches.

Differences from RISC-V — LIMO is derived from RV32I, so it keeps the basic 32-bit load/store design and instruction formats, but uses a smaller, selected instruction set and Sesotho mnemonics suited to the project.