A5 — Encoding and Specification

Register Encoding

Number of registers: 32

Register field size: 5 bits

rd: 5 bits

rs1: 5 bits

rs2: 5 bits

Examples:

| Register | Binary |
| --- | --- |
| R0 | 00000 |
| R1 | 00001 |
| R2 | 00010 |
| R10 | 01010 |
| R31 | 11111 |

5.2 R-Type Format

The R-type format is used for register-to-register arithmetic, logical, and shift operations.

31        25 24    20 19    15 14   12 11     7 6       0

+-----------+--------+--------+-------+--------+---------+

|  funct7   |  rs2   |  rs1   |funct3 |   rd   | opcode  |

+-----------+--------+--------+-------+--------+---------+

7 bits     5 bits   5 bits   3 bits  5 bits    7 bits

Opcode: 0110011

| Mnemonic | funct7 | funct3 |
| --- | --- | --- |
| kpn | 0000000 | 000 |
| tls | 0100000 | 000 |
| le | 0000000 | 111 |
| kp | 0000000 | 110 |
| kpe | 0000000 | 100 |
| stts | 0000000 | 001 |
| stto | 0000000 | 101 |

5.3 I-Type Format

The I-type format is used for immediate operations, word loading, and register-based jumps.

31                    20 19    15 14   12 11     7 6       0

+-----------------------+--------+-------+--------+---------+

|      immediate        |  rs1   |funct3 |   rd   | opcode  |

+-----------------------+--------+-------+--------+---------+

12 bits            5        3       5         7

The immediate field is 12 bits wide and represents signed values from −2048 to +2047.

| Mnemonic | funct3 | Opcode |
| --- | --- | --- |
| kph | 000 | 0010011 |
| leh | 111 | 0010011 |
| kpnh | 110 | 0010011 |
| kpeh | 100 | 0010011 |
| tsl | 010 | 0000011 |
| tlkr | 000 | 1100111 |

5.4 S-Type Format

The S-type format is used for the word-store instruction bl.

31       25 24    20 19    15 14   12 11        7 6       0

+----------+--------+--------+-------+-----------+---------+

|imm[11:5] |  rs2   |  rs1   |funct3 | imm[4:0]  | opcode  |

+----------+--------+--------+-------+-----------+---------+

7 bits    5        5        3        5          7

The immediate field is 12 bits wide and represents signed values from −2048 to +2047.
For bl:
Opcode: 0100011
funct3: 010

5.5 B-Type Format

The B-type format is used for conditional branch instructions.

31 30       25 24    20 19    15 14   12 11      8 7 6       0

+--+-----------+--------+--------+-------+----------+-+---------+

|i12|  i10:5   |  rs2   |  rs1   |funct3 |  i4:1   |i11| opcode|

+--+-----------+--------+--------+-------+----------+-+---------+

The branch immediate is 13 bits including the implicit lowest zero bit. The signed branch offset ranges from −4096 to +4094 bytes, in increments of 2 bytes.

| Mnemonic | funct3 | Opcode |
| --- | --- | --- |
| khl | 000 | 1100011 |
| khsl | 001 | 1100011 |
| khn | 100 | 1100011 |
| khk | 101 | 1100011 |

5.6 J-Type Format

The J-type format is used for tlk and tl.

31 30                  21 20 19                12 11     7 6       0

+--+---------------------+-+--------------------+--------+---------+

|i20|       i10:1        |i11|       i19:12     |   rd   | opcode  |

+--+---------------------+-+--------------------+--------+---------+

The jump immediate is 21 bits including the implicit lowest zero bit. The signed jump offset ranges from −1,048,576 to +1,048,574 bytes, in increments of 2 bytes.

For tlk:
Opcode: 1101111
rd receives the return address.

For tl:
Opcode: 1101111
rd = R0
The return address is therefore discarded.

5.7 Opcode Summary

| Category | Instructions | Format | Opcode |
| --- | --- | --- | --- |
| Arithmetic, Logic and Shift | kpn, tls, le, kp, kpe, stts, stto | R | 0110011 |
| Immediate Operations | kph, leh, kpnh, kpeh | I | 0010011 |
| Load | tsl | I | 0000011 |
| Store | bl | S | 0100011 |
| Conditional Branches | khl, khsl, khn, khk | B | 1100011 |
| Register Jump | tlkr | I | 1100111 |
| Jumps | tlk, tl | J | 1101111 |